Histone Purification Protocol

Liv Jensen, 03/2017 annotated 181025 Lucy Brennan

Adapted from Luger, et al.

- Grow xenopus histones from overnight starter culture in 2XYT at 37˚C.  Induce with 200mM IPTG when OD600 = 0.6; express for 4 hours at 37˚C. (Yield ~50-100mg/L; all buffer volumes given below assume 1L expression culture)

  - Add phosphate salts to growth media (1.7 mM KH2PO4, 9.4 mM K2HPO4 final, follow CSH terrific broth protocol)

  - One day growth:

    - Scape freshly transformed plate (from pLys BL21, Carb/ Cam selection) into 5 mL 2xYT starter, shake at 37 deg for 2 hours

    - Transfer starter to 100 mL 2xYT and grow to OD=0.4

    - Add 25mL of 100mL growth to each liter of 2xYT, shake at 37 deg until OD=0.6

    - Induce at OD=0.6 with 200 mM IPTG (made fresh), grow for 4 hours at 37 deg

- Spin down cells (10min @ 4000RPM), remove supernatant, and freeze pellets at -80˚C.

  - Resuspend cell pellet in 25 mL (per liter pellet) of Buffer B (wash buffer with triton)

  - Flash freeze and store at -20

- Lyse cells:

  - Thaw cell slurry and transfer to metal beaker, on ice, for sonication

  - Sonicate on ice: 75% power, sonicator on 4th floor, 4x 20 seconds: 1 second on/ 1 second off, one minute rest between

  - Spin at 20,000g for 15 min.  Remove supernatant and retain pellet – protein is expressed in inclusion bodies. (Stop point, store pellets at -20)

- Wash inclusion bodies: resuspend inclusion bodies by crushing with a scoopula

  - Wash pellet with 25ml Buffer B, spin at 20,000g for 15min.

  - Repeat step 4a for a total of 3 washes with Buffer B.

  - Wash pellet with 25ml Buffer A, spin at 20,000g for 15min.

  - Repeat step 4c for a total of 3 washes with Buffer A.

- Inclusion bodies can be stored at -20˚C at this point.

- Inclusion body solubilization:

  - Add 0.5ml DMSO, breaking up inclusion body pellet as necessary.

    - Pellet should resuspend easily and look kind of grainy

  - Incubate 1 hr at RT.

  - Add 10ml Buffer C, stiring, RT for 4 hr or overnight.

    - The liquid should be clear after ~4 hours

    - Spin down at 15,000 RPM (JA-20 roter), 4 deg, 20 minutes, load supe into dialysis bag

- Dialyze protein against 3 changes of 1L Buffer D at RT (use 6-8kDa MWCO tubing; histone MW ~11kDa-15kDa).

  - 2 hours per liter minimum, usually I do one overnight (for sanity)

  - There is almost always crash out at this stage, likely due to DNA in the inclusion body

- Ion exchange chromatography:

  - Dilute protein 1:1 with Buffer D, spin down (JA-20 roter), 15,000 RPM, 20 minutes and pass through a 0.2µM syringe-tip filter.

    - Save pellet to assay loss to precipation during dialysis

  - Wash ion exchange column (TOSOH TSKgel SP-5PW, 50ml CV) with Buffer E, then equilibrate with Buffer D for 2 CV at 4ml/min.

  - Load protein onto column at 1 ml/min. Collect flow-through.

    - By keeping the inclusion body solubilization volume to 10 mL, I use a loop as opposed to loading directly through the inlet line

  - Wash column with 1CV Buffer D, 4 mL/min

  - Elute protein in a linear gradient from 0 – 100% Buffer E over 60 min at 4ml/min.

    - 5mL fractions

- Collect and analyze peak fractions by 18% SDS-PAGE.

  - I save the pellets from the Gu-HCl solubilization and after the dialysis to urea to assay histone loss; each pellet is resuspended in 1x TGS buffer and 10uL is loaded on the gel

    - Gu-HCl will precipitate with SDS at room temp, so loading this sample can be tricky. Keep it at hot as possible

- Pool desired fractions and dialyze against water.

- Measure protein concentration by A280, and lyophilize in aliquots of useful size.

  - I ran a BSA standard with dilutions of the histone after dialysis to water

  - To simplify the octamer refolding aliquots of “useful size” are as follows:

    - 1.5 mg H2A and H2B

    - 1.36 mg H3

    - 1 mg H4

    - This allows one to resuspend in 1mL of unfolding buffer and mix all of them together 1:1:1:1 volumetrically, but at a molar ratio or 1.2:1.2:1:1 (H2A:H2B:H3:H4)

- Store lyophilized protein at -80˚C indefinitely.

Buffers:

A:

50mM Tris-HCl pH7.5

100mM NaCl

1mM EDTA

1mM Benzamidine

5mM BME

B:

Buffer A + 1% Triton X-100

C:

6M Guanidinium HCl

50mM Tris-HCl pH7.5

5mM BME

D:

6M Urea (deionized with MB AG 501-X8 (D) Resin from BioRad)

20mM Tris-HCl pH7.5

20mM NaCl

5mM BME

E:

6M Urea (deionized as above)

20mM Tris-HCl pH7.5

2M NaCl

5mM BME

example gels

Left to right: precision plus ladder, BSA standard (lanes 2-8), H4 (lanes 9-11, 1:10 dilution),

H4 (lanes 12-14, 1:100 dilution)

refolded Octamer, unlabeled off the S200 sizing column (FPLC trace not shown)

lanes 8-10 (from the left) pooled and used for mono-nucleosome assemblies